DMMVIEW_G communication protocol

I.      Communication protocol

（1）  General information

§   This communication protocol conforms to SCPI standard

SCPI  Standard Commands for Programmable Instruments

Programmable instrument standard instruction set

§   Version 0.0

§   Baud rate: 9600bps, 8 data bits, 1 stop bit, no check bit

（2）  Communication data block format

l   Communication data frame format

l   Communication command data block

STX: 1 byte ASCII ' #'

Command: 3-byte instruction code _ communication command (see Appendix 1)

Parameter: N-byte ASCII 'XX…XX'

EM: 1-byte ASCII  <cr>

l   Communication return data block

STX: 1 byte ASCII ' #'

Answer: 2-byte instruction code _ return command (see Appendix 1)

Data: N-byte ASCII 'XX…XX'

EM: 1-byte ASCII  <cr>

（3）  Communication command set [COMM]

| Communication command subset | Communication command |
| --- | --- |
| [BASE] | <RST> |
|  | <ONL> |
| [INPUT] | <SEL> |
|  | <RAN> |
|  | <RD?> |
|  | < SC?> |
|  | <SD?> |
|  | <TC?> |
|  | <TI?> |
|  | <TD?> |

Command subset 1: basic command for online communication [BASE]

| < RST > Command |  |
| --- | --- |
| Description | Instrument reset command |
| Command | #<RST><cr> |
| Return | #<ACK><cr> |
| Instruction | §Return to ACK response. §The instrument performs startup operation, which is equivalent to pressing the power button to start. |

| < ONL > command |  |
| --- | --- |
| Description | PC online communication command |
| Command | #<ONL><cr> |
| Return | #<ACK><cr> |
| Instruction | § During instrument input measurement, the PC performs online handshake communication with the instrument. § Enable PC online communication. § If the connection is successful, return to ACK response. |

Command subset 2: instrument input measurement communication command [INPUT]

| < SEL > Command |  |
| --- | --- |
| Description | Auxiliary function selection command |
| Command | #<SEL>x<cr> See Appendix 6 x: function code, 1-byte data |
| Return | §Type 1: deterministic response ACK #<ACK><cr> §Type 2: non-deterministic response NAK #<NAK><cr> |
| Instruction | §Stop the current measurement operation, perform the corresponding measurement operation according to the set parameters, and return ACK response. §If the instrument is in the FREQ measurement function /MEM file, return to NAK response. §If the measurement function code exceeds the range shown in Appendix 6, return NAK response. |

| < RAN > command |  |
| --- | --- |
| Description | Entered measurement range selection command |
| Command | #<RAN>x1 x2<cr> X1: auto range status number, 1-byte data 0x30:manual range 0x31:auto range X2: measurement range number, 1-byte data Detail can be found in Appendix 2. |
| Return | §Type 1: deterministic response ACK #<ACK><cr> §Type 2: non-deterministic response NAK #<NAK><cr> |
| Instruction | §Stop the current measurement operation, perform the corresponding measurement operation according to the set parameters (set parameter 1/ set parameter 2), and return ACK response. §If the measurement range code exceeds the range shown in Appendix 2, return NAK response. |

| <RD?>Command |  |
| --- | --- |
| Description | Reading command of instrument inputting measured data (real-time) |
| Command | #<RD?><cr> |
| Return | §Type 1: deterministic response #<RD> x…x <cr> See Appendices 2 and 5 x…x: input measured data, 26-byte data §Type 2: non-deterministic response NAK #<NAK><cr> |
| Instruction | §The instrument is in the input measurement operation, read the measured data of the current input measurement operation. Return RD response and measured data. §When the meter is in recording and reading operation, return NAK response. |

| <SC?> Command |  |
| --- | --- |
| Description | Reading command of instrument manually recording data size |
| Command | #<SC?><cr> |
| Return | §Type 1: deterministic response #<SC>x…x<cr> x ... x: manually record data, 4-byte data §Type 2: non-deterministic response NAK #<NAK><cr> |
| Instruction | §The meter is in recording and reading operation, and the reading meter records the data manually. Return SC response and record the data volume. §If the meter is in non-recording reading status, return NAK response. |

| <SD?> command |  |
| --- | --- |
| Description | Reading command of instrument manually recording data |
| Command | #<SD?> x…x<cr> x ... x: record number, 4-byte data,data range:1~1000. |
| Return | §Type 1: deterministic response #<SD>x…x<cr> See Appendices 2 and 5 x…x: manually record data, 26-byte data §Type 2: non-deterministic response NAK #<NAK><cr> |
| Instruction | §The instrument is in recording and reading operation, and the reading instrument records data manually. Return SD response and 1 record data unit corresponding to the record number. §If the meter is in non-recording reading status, return NAK response. |

| <TC?>command |  |
| --- | --- |
| Description | Reading command of instrument regularly recording data size |
| Command | #<TC?><cr> |
| Return | §Type 1: deterministic response #<TC> x…x <cr> x…x: record the data size regularly, 4-byte data §Type 2: non-deterministic response NAK #<NAK><cr> |
| Instruction | §The meter is in recording and reading operation, and the reading meter records the data regularly. Return TC response, and record the data size. §If the meter is in non-recording reading status, return NAK response. |

| <TI?> command |  |
| --- | --- |
| Description | Reading command of interval time of timed recording by the instrument |
| Command | #<TI?><cr> |
| Return | §Type 1: deterministic response #<TI>x…x<cr> x... x: time interval of timed recording, 4-byte data §Type 2: non-deterministic response NAK #<NAK><cr> |
| Instruction | §The meter is in recording and reading operation, and the reading meter records the interval time regularly. Return TI response and record the interval time. §If the meter is in non-recording reading status, return NAK response. |

| <TD? > command |  |
| --- | --- |
| Description | Reading command of instrument regularly recording data |
| Command | #<TD?>x…x<cr> x ... x: record number, 4-byte data,data range:1~1000. |
| Return | §Type 1: deterministic response #<TD> x…x<cr> See Appendices 2 and 5 x…x: timed data recording, 26-byte data §Type 2: non-deterministic response NAK #<NAK><cr> |
| Instruction | §The instrument is in recording and reading operation, and the reading meter records the data regularly. Return TD response and 1 record data unit corresponding to the record number. §If the meter is in non-recording reading status, return NAK response. |

Appendix 1:Command Code

l  Send Description

| Command Code | Command |
| --- | --- |
| 0x52 0x53 0x54 | <RST> |
| 0x4F 0x4E 0x4C | <ONL> |
| 0x53 0x45 0x4C | <SEL> |
| 0x52 0x41 0x4E | <RAN> |
| 0x52 0x44 0x3F | <RD?> |
| 0x53 0x43 0x3F | <SC?> |
| 0x53 0x44 0x3F | <SD?> |
| 0x54 0x43 0x3F | <TC?> |
| 0x54 0x49 0x3F | <TI?> |
| 0x54 0x44 0x3F | <TD?> |

l  Answer Description

| Command Code | Command Description |
| --- | --- |
| 0x06 0x00 | <ACK>：Confirm response |
| 0x15 0x00 | <NAK>：Undetermined response |
| 0x52 0x44 | <RD>：data value |

l  ASCII Description

| Chars | Description |
| --- | --- |
| ? | Inquiry |
| <sp> | Space |
| x | One Char |
| x…x | More Chars |
| <cr> | Return |
| <lf> | New Line |

Appendix 2:Measurement Code Description

| Measure |  | Func Code | Range Code | Value Code |
| --- | --- | --- | --- | --- |
| Function |  |  |  | Range |
| ACV | 2V | 0x30 0x30 | 0x30 | ±X.XXXX或FFFFFFF |
|  | 20V |  | 0x31 | ±XX.XXX或FFFFFFF |
|  | 200V |  | 0x32 | ±XXX.XX或FFFFFFF |
|  | 1000V |  | 0x33 | ±XXXX.X或FFFFFFF |
| dBm_V | 2V | 0x30 0x31 | 0x30 | ±XX.XXX或FFFFFFF |
|  | 20V |  | 0x31 | ±XX.XXX或FFFFFFF |
|  | 200V |  | 0x32 | ±XX.XXX或FFFFFFF |
|  | 1000V |  | 0x33 | ±XX.XXX或FFFFFFF |
| ACmV | 200mV | 0x30 0x32 | 0x30 | ±XXX.XX或FFFFFFF |
| dBm_mV | 200mV | 0x30 0x33 | 0x30 | ±XX.XXX或FFFFFFF |
| DCV | 2V | 0x30 0x34 | 0x30 | ±X.XXXX或FFFFFFF |
|  | 20V |  | 0x31 | ±XX.XXX或FFFFFFF |
|  | 200V |  | 0x32 | ±XXX.XX或FFFFFFF |
|  | 1000V |  | 0x33 | ±XXXX.X或FFFFFFF |
| DCmV | 200mV | 0x30 0x35 | 0x30 | ±XXX.XX或FFFFFFF |
| TC | K | 0x30 0x36 | 0x30 | ±XXXX.X或FFFFFFF |
| CONT | 600Ω | 0x30 0x37 | 0x30 | ±XXXX.X或FFFFFFF |
| DIODE | 2V | 0x30 0x38 | 0x30 | ±X.XXXX或FFFFFFF |
| OHM | 200Ω | 0x30 0x39 | 0x30 | ±XXX.XX或FFFFFFF |
|  | 2kΩ |  | 0x31 | ±X.XXXX或FFFFFFF |
|  | 20kΩ |  | 0x32 | ±XX.XXX或FFFFFFF |
|  | 200kΩ |  | 0x33 | ±XXX.XX或FFFFFFF |
|  | 2MΩ |  | 0x34 | ±X.XXXX或FFFFFFF |
|  | 20MΩ |  | 0x35 | ±XX.XXX或FFFFFFF |
|  | 60MΩ |  | 0x36 | ±XXX.XX或FFFFFFF |
| RTD | PT100 | 0x31 0x30 | 0x30 | ±XXXXX.或FFFFFFF |
| CAP | 10nF | 0x31 0x31 | 0x30 | ±XXX.XX或FFFFFFF |
|  | 100nF |  | 0x31 | ±XXXX.X或FFFFFFF |
|  | 1000nF |  | 0x32 | ±XXXXX.或FFFFFFF |
|  | 10µF |  | 0x33 | ±XXX.XX或FFFFFFF |
|  | 100µF |  | 0x34 | ±XXXX.X或FFFFFFF |
|  | 1000µF |  | 0x35 | ±XXXXX.或FFFFFFF |
|  | 10mF |  | 0x35 | ±XXX.XX或FFFFFFF |
|  | 100mF |  | 0x36 | ±XXXX.X或FFFFFFF |
| FREQ DUTY | 10Hz | 0x31 0x32 | 0x30 | ±X.XXXX或FFFFFFF |
|  | 100Hz |  | 0x31 | ±XX.XXX或FFFFFFF |
|  | 1000Hz |  | 0x32 | ±XXX.XX或FFFFFFF |
|  | 10kHz |  | 0x33 | ±X.XXXX或FFFFFFF |
|  | 100kHz |  | 0x34 | ±XX.XXX或FFFFFFF |
|  | 1000kHz |  | 0x35 | ±XXX.XX或FFFFFFF |
|  | 10MHz |  | 0x36 | ±X.XXXX或FFFFFFF |
|  | 占空比 |  | / | ±XXX.XX或FFFFFFF |
| DCuA | 200uA | 0x31 0x33 | 0x30 | ±XXX.XX或FFFFFFF |
|  | 2000uA |  | 0x31 | ±XXXX.X或FFFFFFF |
| DCmA | 20mA | 0x31 0x34 | 0x30 | ±XX.XXX或FFFFFFF |
|  | 200mA |  | 0x31 | ±XXX.XX或FFFFFFF |
| DCA | 2A | 0x31 0x35 | 0x30 | ±X.XXXX或FFFFFFF |
|  | 10A |  | 0x31 | ±XX.XXX或FFFFFFF |
| ACuA | 200uA | 0x31 0x36 | 0x30 | ±XXX.XX或FFFFFFF |
|  | 2000uA |  | 0x31 | ±XXXX.X或FFFFFFF |
| ACmA | 20mA | 0x31 0x37 | 0x30 | ±XX.XXX或FFFFFFF |
|  | 200mA |  | 0x31 | ±XXX.XX或FFFFFFF |
| ACA | 2A | 0x31 0x38 | 0x30 | ±X.XXXX或FFFFFFF |
|  | 10A |  | 0x31 | ±XX.XXX或FFFFFFF |
| PEAK_DCmV | 200mV | 0x31 0x39 | 0x30 | ±XXXX.X或FFFFFFF |
| PEAK_DCV | 2V | 0x32 0x30 | 0x30 | ±XX.XXX或FFFFFFF |
|  | 20V |  | 0x31 | ±XXX.XX或FFFFFFF |
|  | 200V |  | 0x32 | ±XXXX.X或FFFFFFF |
|  | 1000V |  | 0x33 | ±XXXXX.或FFFFFFF |
| PEAK_DCuA | 200uA | 0x32 0x31 | 0x30 | ±XXXX.X或FFFFFFF |
|  | 2000uA |  | 0x31 | ±XXXXX.或FFFFFFF |
| PEAK_DCmA | 20mA | 0x32 0x32 | 0x30 | ±XXX.XX或FFFFFFF |
|  | 200mA |  | 0x31 | ±XXXX.X或FFFFFFF |
| PEAK_DCA | 2A | 0x32 0x33 | 0x30 | ±XX.XXX或FFFFFFF |
|  | 10A |  | 0x31 | ±XXX.XX或FFFFFFF |
| ACmV+HZ | 10Hz | 0x32 0x34 | 0x30 | ±X.XXXX或FFFFFFF |
|  | 100Hz |  | 0x31 | ±XX.XXX或FFFFFFF |
|  | 1000Hz |  | 0x32 | ±XXX.XX或FFFFFFF |
|  | 10kHz |  | 0x33 | ±X.XXXX或FFFFFFF |
|  | 100kHz |  | 0x34 | ±XX.XXX或FFFFFFF |
|  | 1000kHz |  | 0x35 | ±XXX.XX或FFFFFFF |
|  | 10MHz |  | 0x36 | ±X.XXXX或FFFFFFF |
|  | 200mV |  | 0x30 | ±XXX.XX或FFFFFFF |
| ACV+HZ | 2V | 0x32 0x35 | 0x30 | ±X.XXXX或FFFFFFF |
|  | 20V |  | 0x31 | ±XX.XXX或FFFFFFF |
|  | 200V |  | 0x32 | ±XXX.XX或FFFFFFF |
|  | 1000V |  | 0x33 | ±XXXX.X或FFFFFFF |
| ACuA+HZ | 200uA | 0x32 0x36 | 0x30 | ±XXX.XX或FFFFFFF |
|  | 2000uA |  | 0x31 | ±X.XXXX或FFFFFFF |
| ACmA+HZ | 20mA | 0x32 0x37 | 0x30 | ±XX.XXX或FFFFFFF |
|  | 200mA |  | 0x31 | ±XXX.XX或FFFFFFF |
| ACA+HZ | 2A | 0x32 0x38 | 0x30 | ±XXXX.X或FFFFFFF |
|  | 10A |  | 0x31 | ±XXX.XX或FFFFFFF |
| ACmV+DCmV | 200mV | 0x32 0x39 | 0x30 | ±XXX.XX或FFFFFFF |
| ACV+DCV | 2V | 0x33 0x30 | 0x30 | ±X.XXXX或FFFFFFF |
|  | 20V |  | 0x31 | ±XX.XXX或FFFFFFF |
|  | 200V |  | 0x32 | ±XXX.XX或FFFFFFF |
|  | 1000V |  | 0x33 | ±XXXX.X或FFFFFFF |
| ACuA+DCuA | 200uA | 0x33 0x31 | 0x30 | ±XXX.XX或FFFFFFF |
|  | 2000uA |  | 0x31 | ±X.XXXX或FFFFFFF |
| ACmA+DCmA | 20mA | 0x33 0x32 | 0x32 | ±XX.XXX或FFFFFFF |
|  | 200mA |  | 0x33 | ±XXX.XX或FFFFFFF |
| ACA+DCA | 2A | 0x33 0x33 | 0x34 | ±X.XXXX或FFFFFFF |
|  | 10A |  | 0x35 | ±XX. XXX或FFFFFFF |
| MEM | / | 0x33 0x34 | / | / |

Appendix 5:Information structure of communication data unit

| Measure Func (Base) | Function (2byte) | Range1 (1byte) | Data1 (7bytes) | Range2 (1byte) | Data2 (7bytes) | Range3 (1byte) | Data3 (7bytes) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ACV | 0x30 0x30 | 0x30～0x33 | ACV |  |  |  |  |
| dBm_V | 0x30 0x31 | 0x30~0x33 | dBm | 0x30～0x33 | ACV |  | dBm_REF |
| ACmV | 0x30 0x32 | 0x30 | ACmV |  |  |  |  |
| dBm_mV | 0x30 0x33 | 0x30 | dBm | 0x30 | ACmV |  | dBm_REF |
| DCV | 0x30 0x34 | 0x30~0x33 | DCV |  |  |  |  |
| DCmV | 0x30 0x35 | 0x30 | DCmV |  |  |  |  |
| TC_K | 0x30 0x36 | 0x30 | TEMP | 0x30 | DCmV | ℃/℉ | RJC |
| CONT | 0x30 0x37 | 0x30 | CONT |  |  |  |  |
| DIODE | 0x30 0x38 | 0x30 | DIODE |  |  |  |  |
| OHM | 0x30 0x39 | 0x30~0x36 | OHM |  |  |  |  |
| RTD | 0x31 0x30 | 0x30 | TEMP | 0x30 | OHM | ℃/℉ |  |
| CAP | 0x31 0x31 | 0x30~0x37 | CAP |  |  |  |  |
| FREQ/DUTY | 0x31 0x32 | 0x30~0x36 | FREQ | / | DUTY |  |  |
| DCuA | 0x31 0x33 | 0x30~0x31 | DCuA |  |  |  |  |
| DCmA | 0x31 0x34 | 0x30~0x31 | DCmA |  |  |  |  |
| DCA | 0x31 0x35 | 0x30~0x31 | DCA |  |  |  |  |
| ACuA | 0x31 0x36 | 0x30~0x31 | ACuA |  |  |  |  |
| ACmA | 0x31 0x37 | 0x30~0x31 | ACmA |  |  |  |  |
| ACA | 0x31 0x38 | 0x30~0x31 | ACA |  |  |  |  |
| PEAK _DCmV | 0x31 0x39 | 0x30 | PEAK_MAX | 0x30 | PEAK_MIN |  |  |
| PEAK _DCV | 0x32 0x30 | 0x30~0x33 | PEAK_MAX | 0x30~0x33 | PEAK_MIN |  |  |
| PEAK_DCuA | 0x32 0x31 | 0x30~0x31 | PEAK_MAX | 0x30~0x31 | PEAK_MIN |  |  |
| PEAK_DCmA | 0x32 0x32 | 0x30~0x31 | PEAK_MAX | 0x30~0x31 | PEAK_MIN |  |  |
| PEAK_DCA | 0x32 0x33 | 0x30~0x31 | PEAK_MAX | 0x30~0x31 | PEAK_MIN |  |  |
| ACmV+HZ | 0x32 0x34 | 0x30~0x36 | FREQ | / | DUTY | 0x30 | ACmV |
| ACV+HZ | 0x32 0x35 | 0x30~0x36 | FREQ | / | DUTY | 0x30~0x33 | ACV |
| ACuA+HZ | 0x32 0x36 | 0x30~0x36 | FREQ | / | DUTY | 0x30~0x31 | ACuA |
| ACmA+HZ | 0x32 0x37 | 0x30~0x36 | FREQ | / | DUTY | 0x30~0x31 | ACmA |
| ACA+HZ | 0x32 0x38 | 0x30~0x36 | FREQ | / | DUTY | 0x30~0x31 | ACA |
| ACV+DCmV | 0x32 0x39 | 0x30 | AC+DC | 0x30 | ACV | 0x30 | DCV |
| ACV+DCV | 0x33 0x30 | 0x30~0x33 | AC+DC | 0x30~0x33 | ACV | 0x30~0x33 | DCV |
| ACA+DCuA | 0x33 0x31 | 0x30~0x31 | AC+DC | 0x30~0x31 | ACI | 0x30~0x31 | DCuA |
| ACA+DCmA | 0x33 0x32 | 0x30~0x31 | AC+DC | 0x30~0x31 | ACI | 0x30~0x31 | DCmA |
| ACA+DCA | 0x33 0x33 | 0x30~0x31 | AC+DC | 0x30~0x31 | ACI | 0x30~0x31 | DCA |
| VFC | 0x33 0x34 | 0x30 | VFC |  |  |  |  |

Appendix 6 : Description of Measurement Select Function Code

| SW Func | Function Code |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0x30 | 0x31 | 0x32 | 0x33 | 0x34 |
| ACV | ACV | dBm | AC+Hz | VFC | / |
| ACmV | ACmV | dBm | AC+Hz | / | / |
| DCV | DCV | AC+DC | PEAK_DCV | / | / |
| DCmV | DCmV | TC | AC+DC | PEAK_DCmV | / |
| CONT | CONT | DIODE | / | / | / |
| OHM | OHM | RTD | CAP | / | / |
| FREQ | / | / | / | / | / |
| DCuA | DCuA | AC+DC | PEAK_DCuA | ACuA | AC+Hz |
| DCmA | DCmA | AC+DC | PEAK_DCmA | ACmA | AC+Hz |
| DCA | DCA | AC+DC | PEAK_DCA | ACA | AC+Hz |

charset="utf-8"